# Assignment  - 12

Q1. Write the logic of AND Gate using MatLab Script.

function[out] = And1(a,b)

out = a & b;

disp(out);

end

Q2. Write the logic of OR Gate using MatLab Script.

function[out] = OR1(a,b)

out = a | b;

disp(out)

end

Q3. Write the logic of NAND Gate using MatLab Script.

function[out] = NAND1(a,b)

out = ~(a&b);

disp(out)

end

Q4. Write the logic of NOR Gate using MatLab Script.

function[out] = NOR1(a,b)

out = ~(a|b);

disp(out)

end

Q5. Write the logic of XOR Gate using MatLab Script.

function[out] = XOR1(a,b)

out = (a & ~b) | (~a & b);

disp(out)

end

Q6. Write the logic of Half-Adder using MatLab Script.

function[s,c] = HA1(a,b)

s = (a & ~b) | (~a & b);

c = a & b;

disp(s)

disp(c)

end

Q7. Write the logic of Full-Adder using MatLab Script.

function[s,cout] = FA1(a,b,cin)

temp = (a & ~b) | (~a & b);

s = (temp & ~cin) | (~temp & cin);

cout = (a & b) | (b & cin) | (a & cin);

disp(s)

disp(cout)

end

Q8. Implement Logical Right-shift using MatLab Script.

function[out] = LRS(a,x)

out = bitshift(a,-x);

disp(out);

end

Q9. Implement Logical Left-shift using MatLab Script.

function[out] = LLS(a,x)

out = bitshift(a,x);

disp(out);

end

Q10. Implement Arithmetic Right-shift using MatLab Script.

function[out] = ARS(a,x)

out = bitshift(a, -x);

disp(out);

end

Q11. Implement Rotate-Left with carry using MatLab Script.

function[out] = RLWC(a)

out = bitshift(a,1) | bitshift(a,-7);

disp(out);

end

Q12. Implement Rotate-Left without carry using MatLab Script.

function[out] = RLWOC(a)

out = bitshift(a,1) | bitshift(a,-7);

disp(out);

end