Assignment – 4

Q1. Write an Assembly program to add two numbers and store the 8 bit sum in any general purpose register.

CODE:

MVI A, 07H
MVI B, 05H
ADD B
MOV C, A
HLT

+-------+----------+------------+---------------+----------------+----------+

|Address|Mnemonics |Machine Code|Addressing Mode|Instruction Type|Operation |

+-------+----------+------------+---------------+----------------+----------+

|4200   |MVI A, 07H|3E 07       |Immediate      |2 bytes         |A <- 07H  |

|4202   |MVI B, 05H|06 05       |Immediate      |2 bytes         |B <- 05H  |

|4204   |ADD B     |80          |Register       |1 bytes         |A <- A + B|

|4205   |MOV C, A  |4F          |Register       |1 bytes         |C <- A    |

|4206   |HLT       |76          |               |1 bytes         |HALT      |

+-------+----------+------------+---------------+----------------+----------+

Q2. Write an Assembly program to add two numbers and store the 8 bit sum in any Memory location.

CODE:

MVI A, 07H
MVI B, 05H
ADD B
STA 0000H
HLT

+-------+----------+------------+---------------+----------------+------------+

|Address|Mnemonics |Machine Code|Addressing Mode|Instruction Type|Operation   |

+-------+----------+------------+---------------+----------------+------------+

|4200   |MVI A, 07H|3E 07       |Immediate      |2 bytes         |A <- 07H    |

|4202   |MVI B, 05H|06 05       |Immediate      |2 bytes         |B <- 05H    |

|4204   |ADD B     |80          |Register       |1 bytes         |A <- A + B  |

|4205   |STA 0000H |32 00 00    |Direct         |3 bytes         |[0000H] <- A|

|4208   |HLT       |76          |               |1 bytes         |HALT        |

+-------+----------+------------+---------------+----------------+------------+

Q3. Write an Assembly program to subtract two numbers and store the result in any Memory location.

CODE:

MVI A, 7H
MVI B, 5H
SUB B
STA 0000H
HLT

+-------+----------+------------+---------------+----------------+------------+

|Address|Mnemonics |Machine Code|Addressing Mode|Instruction Type|Operation   |

+-------+----------+------------+---------------+----------------+------------+

|4200   |MVI A, 07H|3E 07       |Immediate      |2 bytes         |A <- 07H    |

|4202   |MVI B, 05H|06 05       |Immediate      |2 bytes         |B <- 05H    |

|4204   |SUB B     |90          |Register       |1 bytes         |A <- A - B  |

|4205   |STA 0000H |32 00 00    |Direct         |3 bytes         |[0000H] <- A|

|4208   |HLT       |76          |               |1 bytes         |HALT        |

+-------+----------+------------+---------------+----------------+------------+