# Assignment – 05

Q1. An assembly program to add two numbers and store the 8 bit sum in any general purpose register.

Ans: MVI A,20H;

MVI B,18H;

ADD B;

MOV C,A;

HLT;

Listing:

Q2. An assembly program to add two numbers and store the 8 bit sum in any Memory location.

Ans: MVI A,20H;

MVI B,18H;

ADD B;

STA 2020H;

HLT;

Listing:

Q3. An assembly program to subtract two numbers and store the result in any general purpose resister.

Ans: MVI A,25H;

MVI B,15H;

SUB B;

MOV C,A;

HLT;

Listing:

Q4. An assembly program to subtract two numbers and store the result in any Memory location . Ans:

MVI A,25H;

MVI B,15H;

SUB B;

STA 2000H;

HLT;

Listing:

Q5. Writeaprogramin8085assemblylanguagetoperformthefollowing operations: a. Initialize Register B with some data in Hex form b. Initialize Register C with some data in Hex form c. Initialize Register D with some data in Hex form d. Add contents of register B and D e. If the sum is zero then: - increment the content of register C by 1 else – decrement the content of register C by1

Ans: MVI B,23H;

MVI C,01H;

MVI D,17H;

ADD B;

ADD D;

JZ ZERO;

DCR C;

JMP END;

ZERO: INR C;

END: HLT;

Listing:

Q6. Write a program in 8085 assembly language to perform the following operations: f. Initialize Register B with 80H g. Initialize Register D with 84H h. Add contents of register B and D i. If the add operation generate an carry then: - Store the carry bit in memory location 2020 and 8-bit sum in 2021 memory location. else - Store only the 8-bit sum in 2021 memory location.

Ans: MVI B,80H;

MVI D,84H;

MOV A,B;

ADD D;

JNC NO_CARRY;

MVI A,01H;

STA 2020H;

MOV A,B;

ADD D;

STA 2021H;

JMP END;

NO_CARRY: STA 2021H;

END: HLT;

Listing: