Assignment – 5

Q1. Write a program in 8085 assembly language to perform the following operations:
    a.  Initialize Register B with some data in Hex form
    b.  Initialize Register C with some data in Hex form
    c.  Initialize Register D with some data in Hex form
    d.  Add contents of register B and D
    e.  If the sum is zero then:
             - increment the content of register C by 1
         else
             - decrement the content of register C by 1

MVI B, 15H
MVI C, 30H
MVI D, 45H
ADD B
ADD D
JNZ else
INR C
HLT
else: DCR C
HLT

Listing:

Q2. Write a program in 8085 assembly language to perform the following operations:
    a.  Initialize Register B with 80H
    b.  Initialize Register D with 84H
    c.  Add contents of register B and D
    d.  If the add operation generate an carry then:
              - store the carry bit in memory location 2021H and 8-bit sum in 2020H memory location.
          else
              - store only the 8-bit sum in 2021H memory location.

MVI B, 80H
MVI D, 84H
XRA A
ADD B
ADD D
JNC else
STA 2020H

MVI A, 01H
else: STA 2021H
HLT

- Listing:

Q3. An assembly program to add two 8-bit Hexadecimal numbers and store result in decimal form only which will
       be a 16-bits number.
       Sample data:
       A: 84H
       B: 75H
      Result = 84H + 75H = F9H = 159D
      The result stored at 2020H and 2021H.

MVI A, 84H
MVI B, 75H
ADD B
DAA
STA 2021H
XRA A
ADC A
MVI A, 01H
STA 2020H
HLT

+-------+----------+------------+---------------+----------------+--------------+

|Address|Mnemonics |Machine Code|Addressing Mode|Instruction Type|Operation     |

+-------+----------+------------+---------------+----------------+--------------+

|4200   |MVI A, 84H|3E 84       |Immediate      |2 bytes         |A <- 84H      |

|4202   |MVI B, 75H|06 75       |Immediate      |2 bytes         |B <- 75H      |

|4204   |ADD B     |80          |Register       |1 bytes         |A <- A + B    |

|4205   |DAA       |27          |Register       |1 bytes         |A -> BCD      |

|4206   |STA 2021H |32 21 20    |Direct         |3 bytes         |[2021H] <- A  |

|4209   |XRA A     |AF          |Register       |1 bytes         |A <- A ^ A    |

|420A   |ADC A     |8F          |Register       |1 bytes         |A <- A + A + C|

|420B   |MVI A, 01H|3E 01       |Immediate      |2 bytes         |A <- 01H      |

|420D   |STA 2020H |32 20 20    |Direct         |3 bytes         |[2020H] <- A  |

|4210   |HLT       |76          |               |1 bytes         |HALT          |

+-------+----------+------------+---------------+----------------+--------------+

Q4. An assembly program to multiply two numbers:

XRA A
MVI B, 03H
MVI C, 06H

loop: ADD B
DCR C
JNZ loop
STA 0000H
HLT

+-------+-----------+------------+---------------+----------------+-------------------+

|Address|Mnemonics  |Machine Code|Addressing Mode|Instruction Type|Operation          |

+-------+-----------+------------+---------------+----------------+-------------------+

|4200   |XRA A      |AF          |Register       |1 bytes         |A <- A ^ A         |

|4201   |MVI B, 03H |06 03       |Immediate      |2 bytes         |B <- 03H           |

|4203   |MVI C, 06H |0E 06       |Immediate      |2 bytes         |C <- 06H           |

|4205   |loop: ADD B|80          |Register       |1 bytes         |A <- A + B         |

|4206   |DCR C      |0D          |Register       |1 bytes         |C <- C - 1         |

|4207   |JNZ loop   |C2 05 42    |Direct         |3 bytes         |if Z=0, PC <- 4205H|

|420A   |STA 0000H  |32 00 00    |Direct         |3 bytes         |[0000H] <- A       |

|420D   |HLT        |76          |               |1 bytes         |HALT               |

+-------+-----------+------------+---------------+----------------+-------------------+

Q5. Program to find square of 8-bit number.

XRA A
MVI B, 07H
MOV C, B
loop: ADD B
DCR C
JNZ loop
STA 0000H
HLT

+-------+-----------+------------+---------------+----------------+-------------------+

|Address|Mnemonics  |Machine Code|Addressing Mode|Instruction Type|Operation          |

+-------+-----------+------------+---------------+----------------+-------------------+

|4200   |XRA A      |AF          |Register       |1 bytes         |A <- A ^ A         |

|4201   |MVI B, 07H |06 07       |Immediate      |2 bytes         |B <- 03H           |

|4203   |MOV C, B   |48          |Register       |1 bytes         |C <- B             |

|4204   |loop: ADD B|80          |Register       |1 bytes         |A <- A + B         |

|4205   |DCR C      |0D          |Register       |1 bytes         |C <- C - 1         |

|4206   |JNZ loop   |C2 05 42    |Direct         |3 bytes         |if Z=0, PC <- 4205H|

|4209   |STA 0000H  |32 00 00    |Direct         |3 bytes         |[0000H] <- A       |

|420C   |HLT        |76          |               |1 bytes         |HALT               |

+-------+-----------+------------+---------------+----------------+-------------------+

Q6. Program to find sum of digit of an 8-bit number.

MVI B, 69H
MOV A, B
ANI 0FH
MOV C, A
MOV A, B

MVI D, 04H

loop: RRC
DCR D
JNZ loop
ANI 0FH
ADD C
STA 0000H
HLT

+-------+----------+------------+---------------+----------------+-------------------+

|Address|Mnemonics |Machine Code|Addressing Mode|Instruction Type|Operation          |

+-------+----------+------------+---------------+----------------+-------------------+

|4200   |MVI B, 69H|06 69       |Immediate      |2 bytes         |B <- 69H           |

|4202   |MOV A, B  |78          |Register       |1 bytes         |A <- B             |

|4203   |ANI 0FH   |E6 0F       |Immediate      |2 bytes         |A <- A & 0FH       |

|4205   |MOV C, A  |4F          |Register       |1 bytes         |C <- A             |

|4206   |MOV A, B  |78          |Register       |1 bytes         |A <- B             |

|4207   |MVI D, 04H|16 04       |Immediate      |2 bytes         |D <- 04H           |

|4209   |loop: RRC |0F          |Register       |1 bytes         |right rotate A     |

|420A   |DCR D     |15          |Register       |1 bytes         |D <- D - 1         |

|420B   |JNZ loop  |C2 09 42    |Direct         |3 bytes         |if Z=0, PC <- 4209H|

|420E   |ANI 0FH   |E6 0F       |Immediate      |2 bytes         |A <- A & 0FH       |

|4210   |ADD C     |81          |Register       |1 bytes         |A <- A + C         |

|4211   |STA 0000H |32 00 00    |Direct         |3 bytes         |[0000H] <- A       |

|4214   |HLT       |76          |               |1 bytes         |HALT               |

+-------+----------+------------+---------------+----------------+-------------------+