Assignment – 06

Q1. An assembly program to add two 8-bit Hexadecimal numbers and store result in decimal form only which will be a 16-bits number.

Code:

MVI A,45H;

MVI B,38H;

ADD B;

DAA;

STA 2002H;

MVI A,00H;

ADC A;

STA 2003H;

HLT;

Listing:

Q2. An assembly program to multiply two numbers.

Code: MVI B,05H;

MVI C,06H;

MVI A,00H;

LOOP: ADD C;

DCR B;

JNZ LOOP;

HLT;

Listing:

Q3. Program to find square of 8-bit number.

Code:

MVI B,05H;

MOV C,B;

MVI A,00H;

LOOP: ADD C;

DCR B;

JNZ LOOP;

HLT;

Listing:

Q4. Program to find sum of digit of an 8-bit number.

Code:

MVI A,45H;

MOV B,A;

ANI 0FH;

MOV C,A;

MOV A,B;

ANI 0F0H;

MVI D,04H;

LOOP: RRC;

DCR D;

JNZ LOOP;

ADD C;

HLT;

Listing:

Q5. An assembly program to find the largest number from the given array.

Code:

LXI H,2000H;

MOV C,M;

INX H;

MOV A,M;

DCR C;

LOOP: INX H;

CMP M;

JC UPDATE;

NEXT: DCR C;

JNZ LOOP;

STA 2006H;

HLT;

UPDATE: MOV A,M;

JMP NEXT;

Listing: