Assignment – 6

Q1. Assembly Program to Find Largest of Two 8-bit Numbers.

MVI A, 69H
MVI B, 96H
CMP B
JP large
MOV A, B
large: STA 0000H
HLT

+-------+----------------+------------+---------------+----------------+------------------+

|Address|Mnemonics       |Machine Code|Addressing Mode|Instruction Type|Operation         |

+-------+----------------+------------+---------------+----------------+------------------+

|4200   |MVI A, 69H      |3E 69       |Immediate      |2 bytes         |A <- 69H          |

|4202   |MVI B, 96H      |06 96       |Immediate      |2 bytes         |B <- 96H          |

|4204   |CMP B           |B8          |Register       |1 bytes         |flag <- A − B     |

|4205   |JP large        |F2 09 42    |Direct         |3 bytes         |if S=0, PC ← 4209H|

|4208   |MOV A, B        |78          |Register       |1 bytes         |A <- B            |

|4209   |large: STA 0000H|32 00 00    |Direct         |3 bytes         |[0000H] <- A      |

|420C   |HLT             |76          |               |1 bytes         |HALT              |

+-------+----------------+------------+---------------+----------------+------------------+

Q2. Assembly Program to Find Smallest of Two 8-bit Numbers.

MVI A, 69H
MVI B, 96H
CMP B
JM small
MOV A, B
small: STA 0000H
HLT

+-------+----------------+------------+---------------+----------------+------------------+

|Address|Mnemonics       |Machine Code|Addressing Mode|Instruction Type|Operation         |

+-------+----------------+------------+---------------+----------------+------------------+

|4200   |MVI A, 69H      |3E 69       |Immediate      |2 bytes         |A <- 69H          |

|4202   |MVI B, 96H      |06 96       |Immediate      |2 bytes         |B <- 96H          |

|4204   |CMP B           |B8          |Register       |1 bytes         |flag <- A − B     |

|4205   |JM small        |FA 09 42    |Direct         |3 bytes         |if S=1, PC ← 4209H|

|4208   |MOV A, B        |78          |Register       |1 bytes         |A <- B            |

|4209   |small: STA 0000H|32 00 00    |Direct         |3 bytes         |[0000H] <- A      |

|420C   |HLT             |76          |               |1 bytes         |HALT              |

+-------+----------------+------------+---------------+----------------+------------------+

Q3. WAP to find the largest element of the array.
    +------+--------+-------------+
    |Sl. No|Location|Decimal Value|
    +------+--------+-------------+
    |1     |2100H   |05           |
    |2     |2101H   |31           |
    |3     |2102H   |35           |
    |4     |2103H   |46           |
    |5     |2104H   |29           |
    |6     |2105H   |12           |
    +------+--------+-------------+

XRA A
MVI C, 05H
LXI H, 2100H
loop: MOV B, M
CMP B
JP continue
MOV A, B
continue: INX H
DCR C
JNZ loop
INX H
MOV M, A
HLT

+-------+---------------+------------+---------------+----------------+--------------------+

|Address|Mnemonics      |Machine Code|Addressing Mode|Instruction Type|Operation           |

+-------+---------------+------------+---------------+----------------+--------------------+

|4200   |XRA A          |AF          |Register       |1 bytes         |A <- A ^ A          |

|4201   |MVI C, 05H     |0E 05       |Immediate      |2 bytes         |C <- 05H            |

|4203   |LXI H 2100H    |21 00 21    |Immediate      |3 bytes         |HL <- 2100H         |

|4206   |loop: MOV B, M |46          |Indirect       |1 bytes         |B <- [HL]           |

|4207   |CMP B          |B8          |Register       |1 bytes         |flag <- A - B       |

|4208   |JP continue    |F2 0C 42    |Direct         |3 bytes         |if S=0, PC <- 420CH |

|420B   |MOV A, B       |78          |Register       |1 bytes         |A <- B              |

|420C   |continue: INX H|23          |Register       |1 bytes         |HL <- HL + 1        |

|420D   |DCR C          |0D          |Register       |1 bytes         |C <- C - 1          |

|420E   |JNZ loop       |C2 06 42    |Direct         |3 bytes         |if Z=0, PC <- 4206H |

|4211   |INX H          |23          |Register       |1 bytes         |HL <- HL + 1        |

|4212   |MOV M, A       |77          |Indirect       |1 bytes         |[HL] <- A           |

|4213   |HLT            |76          |               |1 bytes         |HALT                |

+-------+---------------+------------+---------------+----------------+--------------------+

Q4. Write a program to sort given 10 numbers from memory location 2100H to 2109H in the ascending order.

+------+--------+-------------+
    |Sl. No|Location|Decimal Value|
    +------+--------+-------------+
    |1     |2100H   |05           |
    |2     |2101H   |31           |
    |3     |2102H   |35           |
    |4     |2103H   |46           |
    |5     |2104H   |29           |
    +------+--------+-------------+

MVI B, 09H
outer_loop: MVI A, 09H
ADD B
SBI 09H
MOV C, A
LXI H, 2100h
inner_loop: MOV A, M
INX H
MOV D, M
CMP D
JM continue
MOV A, M
DCX H
MOV E, M
MOV M, A
INX H
MOV M, E

continue: DCR C
JNZ inner_loop

DCR B
JNZ outer_loop
HLT

+-------+----------------------+------------+---------------+----------------+-------------------+

|Address|Mnemonics             |Machine Code|Addressing Mode|Instruction Type|Operation          |

+-------+----------------------+------------+---------------+----------------+-------------------+

|4200   |MVI B, 09H            |06 09       |Immediate      |2 bytes         |B <- 09H           |

|4202   |outer_loop: MVI A, 09H|3E 09       |Immediate      |2 bytes         |A <- 09H           |

|4204   |ADD B                 |80          |Register       |1 bytes         |A <- A + B         |

|4205   |SBI 09H               |DE 09       |Immediate      |2 bytes         |A <- A - 09H       |

|4207   |MOV C, A              |4F          |Register       |1 bytes         |C <- A             |

|4208   |LXI H, 2100H          |21 00 21    |Immediate      |3 bytes         |HL <- 2100h        |

|420B   |inner_loop: MOV A, M  |7E          |Indirect       |1 bytes         |A <- [HL]          |

|420C   |INX H                 |23          |Register       |1 bytes         |HL <- HL + 1       |

|420D   |MOV D, M              |56          |Indirect       |1 bytes         |D <- [HL]          |

|420E   |CMP D                 |BA          |Register       |1 bytes         |flag <- A - D      |

|420F   |JM continue           |FA 18 42    |Direct         |3 bytes         |if S=1, PC <- 4218H|

|4212   |MOV A, M              |7E          |Indirect       |1 bytes         |A <- [HL]          |

|4213   |DCX H                 |2B          |Register       |1 bytes         |HL <- HL - 1       |

|4214   |MOV E, M              |5E          |Indirect       |1 bytes         |E <- [HL]          |

|4215   |MOV M, A              |77          |Indirect       |1 bytes         |[HL] <- A          |

|4216   |INX H                 |23          |Register       |1 bytes         |HL <- HL + 1       |

|4217   |MOV M, E              |73          |Indirect       |1 bytes         |[HL] <- E          |

|4218   |continue: DCR C       |0D          |Register       |1 bytes         |C <- C - 1         |

|4219   |JNZ inner_loop        |C2 0B 42    |Direct         |3 bytes         |if Z=0, PC <- 420BH|

|421C   |DCR B                 |05          |Register       |1 bytes         |B <- B - 1         |

|421D   |JNZ outer_loop        |C2 02 42    |Direct         |3 bytes         |if Z=0, PC <- 4202H|

|4220   |HLT                   |76          |               |1 bytes         |HALT               |

+-------+----------------------+------------+---------------+----------------+-------------------+